POLICY & PROCEDURE

UNIVERSITY HOSPITAL	    Prepared by: Alix R. Laguerre                         No. LAB-23I

SUNY HEALTH SCIENCE

CENTER AT BROOKLYN	    Reviewed by: Maria I. Mendez	            Issue Date: 04/06

450 CLARKSON AVENUE

BROOKLYN, NY  11203          Approved by: Peter J. Howanitz, MD      Supersedes: None

Review Date: 04/06

DEPARTMENT OF LABORATORIES

FERN TEST – AMNIOTIC FLUID CRYSTALLIZATION TEST

- Procedure:

Specimen: Vaginal swab obtained from the posterior vaginal pool.

  - Avoid the use of any lubricants or antiseptics

  - Use of a sterile swab and do not touch mucus plug

  - Prepare a thin smear of a glass microscope side by spreading evenly

Testing:

- Allow the slide to air dry – do not apply heat and do not coverslip the slide.

- Examine the fully-dried slide microscopically, using the 10X objective.

- Observe for “fern-like” crystals.  Presence of crystals indicates that the fluid is amniotic fluid.

- Record the results:  Write your name, the date, time and findings on the patient record.

Interfering Substances/limitations:

If present, blood, urine cervical mucus can result in false positive findings.

False negative findings can result from prolonged rupture of membranes (>24hr).

Quality control:

- Follow the procedural instructions exactly.

- There are no external (nor internal) quality controls to be performed at the unit level.

Reference:  Medical Center of Louisiana @ New Orleans

PROVIDER PERFORMED MICROSCOPY PROCEDURES

Purpose:

To ensure that all provider performed Microscopy done at this hospital is carried out according to policy and for the maximum benefit of the patients and care providers.  The testing must be performed under legal, regulatory and professional accreditation guidelines as appropriate.

Policy:

All PPMP done at this hospital is the responsibility of the UHB Director of Clinical Laboratories.

Definitions:

- Provider Performed microscopy Procedures (PPMP):  Procedures that must be performed during the patient’s visit by the provider or by a member of the group practice and can only be performed using Bright-field or phase contrast microscope.  PPMP is a subcategory of moderately complex procedures.

WAIVED AND POINT-OF-CARE TESTING

COMPLEXITY OF TESTING:

|  | Certificate | Requirements |
| --- | --- | --- |
| Waived | Waiver | Valid Certificate. Follow JCAHO/FDA standards and manufacturer’s recommendations. |
| Provider Performed Microscope (Subcategory Moderate) | PPM | Valid PPM Certificate. Subject to less extensive moderate complexity standards of QA, QC, PTM and PT. Inspections may be at random or as a result of a complaint. |
| Moderate | Registration and Certificate Accreditation | Valid Certificate. Follow all applicable standards for QA, QC, PTM, PT and staff qualifications. Inspections are biennial or as a result of a complaint. |
| High | Registration and certificate of accreditation | Valid Certificate. Follow all applicable standards for QA, QC, PTM, PT and staff qualifications. The staff and QC standards are more stringent than in the moderate category. |