SUNY DOWNSTATE MEDICAL CENTER

UNIVERSITY HOSPITAL OF BROOKLYN

POLICY AND PROCEDURE

No:  Section 15A

Page 1 of 3

# Subject: CATHETER ASSOCIATED URINARY

# TRACT INFECTIONS					Original Issue date: 1978

Prepared by:  George Allen, Ph.D., CIC		Supercedes: August 2006

Reviewed/Approved by:				Approval Date: August 2007

Michael Augenbraun, MD, FACP

Distribution:	Infection Control Manual

Issued by:  Infection Control Program

___________________________________________________________________________

I:	Purpose: To provide guidelines for the prevention of urinary catheter associated infections.

II:	Definition: None

III:	Policy: Sterile technique will be utilized for the insertion of urinary catheters

IV:	Role Responsibility: Department head/designee, head nurses will ensure compliance

V:	Procedures/Guidelines:

The urinary tract is a common site of healthcare associated infection (HAI) accounting for 15% of all HAI infections in the critical care area at Downstate Medical Center.  Most of these infections follow instrumentation of the urinary tract, mainly urinary catheterization.  Although not all catheter-associated urinary tract infections can be prevented, it is believed that a large number could be avoided by the proper management of the indwelling catheter.  The following recommendations were developed for the care of patients with temporary indwelling urethral catheters.

Personnel

Only persons (e.g., hospital personnel, family members, or patients themselves) who know the

correct technique of aseptic insertion and maintenance of the catheter should handle catheters.

Catheter Use

Urinary catheters should be inserted only when necessary and left in place only for as long as

necessary.  They should not be used solely for the convenience of patient-care personnel. For

selected patients, other methods of urinary drainage such as condom catheter drainage, supra-

pubic catheterization, and intermittent urethral catheterization, adult disposable diapers pads

can be useful alternatives to indwelling urethral catheterization.

Hand washing

Hand washing should be done immediately before and after any manipulation of the catheter

site or apparatus.  (ICM Section 15D).

Catheter Insertion

Catheters should be inserted using aseptic technique and sterile equipment. Gloves, drape,

sponges, and appropriate antiseptic solution for periurethral cleaning, and a single-use packet

of lubricant jelly should be used for insertion. As small a catheter as possible, consistent with

good drainage, should be used to minimize urethral trauma. Indwelling catheters should be

properly secured after insertion to prevent movement and urethral traction.

Closed Sterile Drainage

A sterile, continuously closed drainage system should be maintained. The catheter and

drainage tube should not be disconnected.  If irrigation is anticipated, 3-way catheters should be

used.

Irrigation

Irrigation should be avoided unless obstruction is anticipated (e.g., as might occur with bleeding

after prostatic or bladder surgery); closed continuous irrigation may be used to prevent

obstruction.  To relieve obstruction due to clots, mucus, or other causes, an intermittent method

of irrigation may be used.  Continuous irrigation of the bladder with antimicrobials has not

proven to be useful and should not be performed as a routine infection prevention measure. If

the catheter becomes obstructed it should be changed.

Specimen Collection

If small volumes of fresh urine are needed for examination, the sampling port should be

cleansed with a disinfectant, and urine then aspirated with a sterile needle and syringe.

Larger volumes of urine for special analyses should be obtained aseptically from the drainage

bag.

Urinary Flow

Unobstructed flow should be maintained (occasionally, it is necessary to temporarily obstruct

the catheter for specimen collection or other medical purposes). To achieve free flow of urine

the catheter and collecting tube should be kept from kinking and the collecting bag should be

emptied regularly using a separate collecting container for each patient (the drainage spigot and

non-sterile collecting container should never come in contact). Poorly functioning or obstructed

catheters should be replaced and collecting bags should always be kept below the level of the

bladder.

Meatal Care

Catheter care shall consist of gently cleansing the perineal area with soap and water during the

patient's daily bath and only removing gross debris from the catheter.  It is important not to

manipulate the catheter more than is absolutely necessary because vigorous routine catheter

care may predispose the patient to HAI.

Catheter Change Interval

Indwelling catheters should not be changed at arbitrary fixed intervals.

Spatial Separation of Catheterized Patients

To minimize the chances of cross-infection, infected and uninfected patients with indwelling

catheters preferably should not share the same room or adjacent beds, if the organism causing

the infection is multi-drug resistant.

VI:	Reason for Revision: Review

VII:	Attachments: None

VIII:	References: CDC, APIC

| Date Reviewed | Revision Required (Check One) |  | Responsible Staff Name and Title |
| --- | --- | --- | --- |
| 8/15/2006 | Yes | Yes | George Allen, Ph.D. Dir. of Infection Control |
| 8/07 | Yes | Yes | G Allen, PhD, CIC, CNOR |
|  | Yes | Yes |  |
|  | Yes | Yes |  |