SUNY DOWNSTATE MEDICAL CENTER

UNIVERSITY HOSPITAL OF BROOKLYN

POLICY AND PROCEDURE

No:  Section 16X

Page 1 of 2

Subject: GUIDELINES FOR INFECTION

CONTROL IN THE PULMONARY

FUNCTION LABORATORY		Original Issue date: 1984

Prepared by:  George Allen, Ph.D., CIC		Supercedes: June 2003

Reviewed/Approved by:				Approval Date: April 2007

Michael Augenbraun, MD, FACP

Distribution:	Infection Control Manual

Issued by:  Infection Control Program

___________________________________________________________________________

I:	Purpose: To provide guidelines to prevent the transmission of infections in patients 	undergoing procedures in the pulmonary function lab.

II:	Definition: None

III:	Policy: Universal/Standard precautions shall be utilized in the care of all patients.

IV:	Role Responsibility: Department head/designee/Medical director/Chief Technician will 	ensure compliance

V:	Procedures/Guidelines:

Patient

Patients in isolation precautions: The floor nurse must notify the Pulmonary Function Lab.  The

procedure should, if possible, be deferred until the patient is out of isolation precautions.

Otherwise, the patient is transported to the department (see ICM Section 5). The procedure

room should be cleaned as for isolation precaution.

Patient's with IV therapy and urinary catheters shall have these systems cared for using aseptic

technique (See ICM 15A, 15B)

Personnel

Must comply with pre-employment and annual health exam requirements and procedures

outlined in the ICM 20.

Personnel shall wash hands before and after patient contact, the alcohol waterless based

product is acceptable if hands are not visibly soiled (See ICM 15D). Personnel shall not eat or

drink, in the lab.

Dress Code

Designated dress codes should be followed in cases of isolation precaution.

Universal Standard Precautions

Universal standard precautions shall be utilized in the care of all patients (ICM Sections 2, 2A)

Materials and Equipment

The machine used must be cleaned and maintained as per manufacturer’s guidelines.

A high level disinfection process at a minimum shall be used for mouthpieces and “nose clips”

between patients. This procedure shall be preceded by washing and followed by a thorough

rinse (OPA policy 15E).

A disposable microbial filter will be attached to the mouthpiece and machine in order

to filter potentially infectious substances and reduce the potential for contamination of

the spirometer.

VI:	Reason for Revision: Review

VII:	Attachments: None

VIII:	References: CDC, APIC

| Date Reviewed | Revision Required (Check One) |  | Responsible Staff Name and Title |
| --- | --- | --- | --- |
| 4/07 | Yes |  | George Allen, Ph.D. Dir. of Infection Control |
|  | Yes | Yes |  |
|  | Yes | Yes |  |
|  | Yes | Yes |  |